ZITA Recommendations

In consideration of the wildlife issues still outstanding before the E&L working group, the importance of addressing these issues appropriately, and the limited time remaining to quantify the resources in REZs, the ZITA work group has been discussing contingency recommendations for the Technical Committee, based on some of the options E&L is considering.  The following ZITA work group proposal to the Technical Committee is dependent upon E&L’s final recommendation.

- The QRAs generally stay as they were approved by the Technical Committee, discounting the possible avoid areas that E&L decides. The remaining qualifying resources are what count toward identifying a REZ.

- The qualifying resources, after discounting E&L avoid areas, that can be aggregated within a radius of 100 miles of the centriod and sum up to at least 1,500 MW (or in the case of geothermal, 500 MW), those resources become a REZ.  A single REZ may comprise the remaining qualifying resources of two or more QRAs, as long as the all of the resources of the various QRAs are within a 100 mile radius of each other and meet the 1,500 MW threshold.

- Add the MW potential from biomass, small hydro, and other REZ resources to the total potential remaining from the QRAs.  In some instances, despite the reductions due to avoid areas, this extra MW potential may enable the total resources to exceed the 1,500 MW threshold.  Additionally, new information has been developed on some resources since the QRAs were first identified.   In some cases, this updated information adds to the QRA potential.   Any additional potential will not be counted that is an E&L avoid area.

- Change the discount for solar thermal to 95% (from 96.5%). The areas that are removed due to being classified as an avoid areas raises the certainty of what can and cannot be developed ; thus enabling the calculated concentration of solar development in an area to be somewhat greater.